Using CMU SNMP 3.4 with the PortMaster

Obtain the file cmu-liv-mib.txt from the eSight Service Center at http://www.esight.com.

The file cmu-liv-mib.txt contains the original cmu-snmp mib.txt with the Livingston specific extensions appended to the file.

If CMU SNMP is installed and working correctly, Rename any existing /usr/local/lib/mib.txt to /usr/local/lib/mib.orig (The pre-compiled version for Linux may require the file to be placed in /usr/lib).

Copy cmu-liv-mib.txt into the /usr/lib directory and rename it mib.txt.

To test that everything works, type:

snmpwalk  public ".1.3.6.1.4.1.307"